[image: image1.jpg]


     


SUGGESTED LANGUAGE to SPECIFY 



[image: image2.jpg]



AXIS SOIL AMENDMENTS and PLAY BALL! INFIELD CONDITIONER

In “GREENBOOK” FORMAT

The following information was developed to assist Landscape Architects who are interested in Specifying AXIS Calcined Diatomaceous Earth as a soil amendment in a variety of Landscape planting and soil conditioning procedures; and for Specifying Play Ball! Infield Conditioner into Infield Mixes of Baseball and Softball Fields.  Each page has information and language intended to be “cut and pasted” into the two sections of the “Greenbook” necessary to specify our products; Section 212 – Landscape and Irrigation Materials, and Section 308 – Landscape and Irrigation Installation.  

As a Landscape Architect, if you do not find the planting approach or soil conditioning instruction that follows the scope and requirements of your project, we would be happy to discuss the particulars with you and provide appropriate language to suit your project. 

Specifications in Greenbook and CSI Formats can be downloaded from our website.
CONTENTS for AMENDING:

Bioswales

















Page 2


Individual Plantings – Amending Existing Backfill 





 


Page 3

Individual Plantings – Backfill Blended Offsite and Placed






Page 4

Rooftop Plantings















Page 5
Ground Cover and Vine Planting



   








Page 6 

Turf Reinforcement for Fire Lanes, Access Lanes and Overflow Parking



Page 7

Turf Root Zone – Amend Existing Soil 











Page 8

Turf Root Zone – New Construction Golf Greens









Page 9

Turf Root Zone -  Sand Base Tees, Fairways or Sportsfields






Page 10

Existing Infield Soil












          

Page 11
Infield Soil Blended Offsite













Page 12
Application Rates for AXIS













Page 13

Application Rates for Play Ball!












Page 14
www.axisplayball.com 
       [image: image3.jpg]KEnviroTech

Soil Solutions, Inc.







866-546-3722
[image: image4.jpg]





AMENDING SOIL for BIOSWALES





Page 2
SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-2.3 Topsoil Preparation and Conditioning


308-2.3.2 Fertilizing and Conditioning Procedures. The planting area shall be brought to finish grade, minus the volume equal to the amount of fertilizers and conditioners to be installed, before spreading the fertilizers or conditioning materials specified.


Fertilizing and conditioning materials shall be mechanically spread at a uniform rate. AXIS shall be applied at the rate of 1,250 pounds per 1,000 square feet.  The quantities of materials necessary for the planting area shall be at the site and shall be verified by delivery tickets furnished to the Engineer before spreading. 


After spreading, the fertilizing and conditioning materials shall be uniformly cultivated into the upper 150 mm (6 inches) of soil by suitable equipment operated in at least two directions approximately at right angles. The resulting soil shall be in a friable condition.

[image: image5.jpg]


      


  AMENDING EXISTING BACKFILL

  


Page 3
for INDIVIDUAL TREE AND SHRUB PLANTINGS
SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment.  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 15% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com,  866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-4 PLANTING.


308-4.5 Tree and Shrub Planting. Planting holes ….. planted. 


Containers ….. hole. 




The native soil ….. of 150 mm (6 inches). 


All planting holes shall be backfilled with 15 percent AXIS “Regular” Calcined Diatomaceous Earth.       


Apply AXIS “Regular” over the excavated dirt at the rates per plant listed below.  Contact supplier for other plant sizes not listed. Mix thoroughly with excavated dirt and other fertilizer, organic material, or biological components before backfilling and during backfilling for a homogenous backfill blend.  


1.) Container/Plant Size


Pounds of AXIS      

Equivalent Measure                
           





4”  
          

  

.37 lbs. 
     



2 cups
 





1 gal. 
            

  
1.5 lbs. 




1.5 quarts
 





2 gal.    
 

  

  3  lbs. 

     


3.0 quarts          





5 gal.  
 

  

7.5 lbs. 




1.25 gallons        





24” shrub 
               

3  lbs. 
      



3.0 quarts       





48” shrub
 
 


15 lbs. 
   



2.5 gallons        





6' tree

          
 
15 lbs. 
   



2.5 gallons        





8' tree

          
 
25 lbs. 
   



1.0 bag 
 





12’ tree

              

60 lbs. 
                          
2.4 bags
 





2" caliper
         
 

50 lbs.               


2.0 bags






3" caliper
         
 

75 lbs. 
   



3.0 bags


After planting, the plant shall be plumb, ….

[image: image6.jpg]


 

AMENDING PREPARED SOIL MIX as BACKFILL

  

Page 4
for TREE AND SHRUB PLANTING
SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment.  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 15% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-4 PLANTING.


308-4.5 Tree and Shrub Planting. Planting holes……. planted.


Containers shall …… hole. 




The native soil …… of 150 mm (6 inches). 


All planting holes shall be backfilled with a prepared soil mix. Soil shall consist of 70 percent of the specified topsoil and 15 percent Type 1, 2, or 3 organic soil amendment, and 15 percent AXIS “Regular” Calcined Diatomaceous Earth.  
1.) To make one cubic yard of prepared soil mix, blend 0.15 yard of AXIS (105 pounds) with 0.7 yard topsoil and 0.15 yard of Type 1, 2, or 3 organic soil amendment.

After planting, the plant shall be plumb, ….
[image: image7.jpg]


 

   
   AMENDING ROOFTOP GARDEN SOIL 



Page 5
for TREES, SHRUBS, and SEEDUMS
SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment.  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-4 PLANTING.


308-4.5 Tree and Shrub Planting. Planting holes……. planted.



Containers shall …… hole. 




The native soil …… of 150 mm (6 inches). 


All planting holes shall be backfilled with a prepared Rooftop mix. Soil shall consist of 70 percent of the specified topsoil and 20 percent Type 1, 2, or 3 organic soil amendment, and 10 percent AXIS “Regular” Calcined Diatomaceous Earth.  



1.) To make one cubic yard of prepared soil mix, blend 0.1 yard of AXIS (140 pounds) with 0.7 yard 

   topsoil and 0.2 yard of Type 1, 2, or 3 organic soil amendment.


After planting, the plant shall be plumb, ….

[image: image8.jpg]





 AMENDING SOIL for GROUND COVER
   



Page 6 
AND VINE PLANTING
SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-4.7 Ground Cover and Vine Planting.  Soil Conditioner AXIS shall be applied at the rate of 675 pounds per 1,000 square feet and incorporated into the soil 3 inches deep.  Soil preparation and fine grading shall be completed prior to ground cover planting.

Ground cover and vines shall be planted in moist soil…. 

[image: image9.jpg]





 
AMENDING SOIL for GRASS PAVERS



Page 7
for FIRE LANES, ACCESS LANES, and OVERFLOW PARKING

SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-2.4 Greensmix Preparation and Conditioning


308-2.3.2 Fertilizing and Conditioning Procedures. All soil blending of topsoil, organic amenment, inorganic amendment, and fertilizer components must be carefully blended an accurate and consistent mix.  


AXIS shall be applied at the rate of 10% by volume, which requires 0.1 yard of AXIS (70 pounds) to be blended with 0.70 yard of topsoil, and 0.2 yard of Type 1, 2 or 3 organic soil amendment to create each cubic yard of soil blend.  The quantities of materials necessary for the planting area shall be at the site and shall be verified by delivery tickets furnished to the Engineer before blending. 

Blended mix shall then be placed in the grass paver cavity or area to the expressed depth of finished mix as indicated in the plans.  


308-4.8 Lawn Planting.


308-4.8.1 General.  Before planting lawn, all specified soil preparation and fine grading shall be completed.    
[image: image10.jpg]



     AMENDING EXISTING SOIL for TURF AREAS and           Page 8
PLANTING  With SEED, SOD, or STOLONS
SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-2.3 Topsoil Preparation and Conditioning


308-2.3.2 Fertilizing and Conditioning Procedures. The planting area shall be brought to finish grade, minus the volume equal to the amount of fertilizers and conditioners to be installed, before spreading the fertilizers or conditioning materials specified.


Fertilizing and conditioning materials shall be mechanically spread at a uniform rate. AXIS shall be applied at the rate of 1,250 pounds per 1,000 square feet.  The quantities of materials necessary for the planting area shall be at the site and shall be verified by delivery tickets furnished to the Engineer before spreading. 


After spreading, the fertilizing and conditioning materials shall be uniformly cultivated into the upper 150 mm (6 inches) of soil by suitable equipment operated in at least two directions approximately at right angles. The resulting soil shall be in a friable condition.


308-4.8 Lawn Planting.


308-4.8.1 General.  Before planting lawn, all specified soil preparation and fine grading shall be completed.    
[image: image11.jpg]







  AMENDING GOLF GREEN 





Page 9
ROOTZONES in NEW CONSTRUCTION

SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment  Soil Conditioner shall be AXIS “Fine” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
142% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-2.4 Greensmix Preparation and Conditioning


308-2.3.2 Fertilizing and Conditioning Procedures. All soil blending of sand, peat, inorganic amendment, and fertilizer components must be blended by professional blending equipment to ensure an accurate and consistent mix.  


AXIS shall be applied at the rate of 10% by volume, which requires 0.1 yard of AXIS (70 pounds) to be blended  with 0.90 yard of sand, and 0.05 yard of peat to create each cubic yard of soil blend.  The quantities of materials necessary for the planting area shall be at the site and shall be verified by delivery tickets furnished to the Engineer before blending. 
Blended mix shall then be placed in the greens cavity or area to the expressed depth of finished mix as indicated in the plans.  

308-4.8 Lawn Planting.


308-4.8.1 General.  Before planting lawn, all specified soil preparation and fine grading shall be completed.    
[image: image12.jpg]




   AMENDING SAND BASE TEES, FAIRWAYS 



Page 10
or SPORTSFIELD ROOTZONES in NEW CONSTRUCTION

SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.6  Inorganic Amendment  Soil Conditioner shall be AXIS “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
114% 


Pore Size




0.1-1.0 micron


pH






7


CEC





27

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-2.4 Greensmix Preparation and Conditioning


308-2.3.2 Fertilizing and Conditioning Procedures. All soil blending of sand, peat, inorganic amendment, and fertilizer components must be blended by professional blending equipment to ensure an accurate and consistent mix.  


AXIS shall be applied at the rate of 10% by volume, which requires 0.1 yard of AXIS (70 pounds) to be blended  with 0.90 yard of sand, and 0.05 yard of peat to create each cubic yard of soil blend.  The quantities of materials necessary for the planting area shall be at the site and shall be verified by delivery tickets furnished to the Engineer before blending. 

Blended mix shall then be placed in the greens cavity or area to the expressed depth of finished mix as indicated in the plans.  


308-4.8 Lawn Planting.


308-4.8.1 General.  Before planting lawn, all specified soil preparation and fine grading shall be completed.    
[image: image13.jpg]





         AMENDING EXISTING INFIELD SOIL for 



Page 11
                      SKINNED AREAS of BASEBALL and SOFTBALL FIELDS



























SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.7  Infield Conditioner
. Infield Conditioner shall be Play Ball! “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
90% 


Bulk Density
 (lbs./cu.ft.) 

25

Minus 80 mesh



0.5%

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212


308-2.3 Topsoil Preparation and Conditioning


308-2.3.2 Fertilizing and Conditioning Procedures. The area shall be brought to finish grade, minus the volume equal to the amount of conditioners to be installed, before spreading the infield conditioning materials specified.


Conditioning materials shall be mechanically spread at a uniform rate. Play Ball! “Regular” shall be applied at the rate of 1,250 pounds per 1,000 square feet.  The quantities of materials necessary for the infield area shall be at the site and shall be verified by delivery tickets furnished to the Engineer before spreading. 


After spreading, the conditioning materials shall be uniformly cultivated into the upper 150 mm (6 inches) of soil by suitable equipment operated in at least two directions approximately at right angles.
The infield shall be compacted to provide a uniform, evenly graded surface according the plans.
www.axisplayball.com 
       [image: image14.jpg]KEnviroTech

Soil Solutions, Inc.







866-546-3722

[image: image15.jpg]





     BLENDING INFIELD SOIL OFFSITE  for 



Page 12
                      SKINNED AREAS of BASEBALL and SOFTBALL FIELDS



SECTION 212 - LANDSCAPE AND IRRIGATION MATERIALS


212-1 LANDSCAPE MATERIALS.


212-1.2 Soil Fertilizing and Conditioning Materials.


212-1.2.7  Infield Conditioner
. Infield Conditioner shall be Play Ball! “Regular” Calcined Diatomaceous Earth, applied at 10% by volume, available from EnviroTech Soil Solutions, Inc., www.axisplayball.com, 866-546-3722.  Soil Conditioner shall contain the following properties:

Opaline Silica (SiO2)


90%


Porosity





82%


Absorption (ASTM F-726)
90% 


Bulk Density
 (lbs./cu.ft.) 

25


Minus 80 mesh



0.5%

SECTION 308 - LANDSCAPE AND IRRIGATION INSTALLATION

308 CROSS REFERENCE

Landscape and Irrigation .............................................212



308-2.4 Infield Mix and Conditioning


308-2.3.2 Fertilizing and Conditioning Procedures. All soil blending of infield components must be blended by professional blending equipment to ensure an accurate and consistent mix.  


PLAY BALL! shall be applied at the rate of 10% by volume, which requires 0.1 yard of PLAY BALL! (67 pounds) to be blended  with 0.9 yards of infield mix to create each cubic yard of soil blend.  The quantities of materials necessary for the infield area shall be at the site specified for blending by the Landscape Architect and/or owner and shall be verified by delivery tickets furnished to the Engineer before blending. 

Blended mix shall then be placed in the infield cavity or area to the expressed depth of finished mix as indicated in the plans.  The infield shall be compacted to provide a uniform, evenly graded surface according the plans.
[image: image16.jpg]






       












  Page 13

General Application Rates 
Turf or Groundcover Areas: Spread AXIS evenly at the rates below and incorporate to the desired depth to equal 10% by volume:



2” deep -   use 450 lbs. per 1,000 sq. ft.



4” deep -   use 900 lbs. per 1,000 sq. ft.



6” deep - use 1,300 lbs. per 1,000 sq. ft.



8” deep - use 1,750 lbs. per 1,000 sq. ft. 

Turf Aeration: Topdressing AXIS after Aerating with 5/8” tines on a 3” x 4” pattern will require 100 to 125 lbs. of AXIS per 1,000 square feet.  We recommend harvesting the cores, and then applying the AXIS with a broadcast or drop spreader.  Using a rake or blower will help get the AXIS into the holes.

DryJect Injection: Aeration patterns are either 3” x 2” or 3” x 4”.  

3 x 2 spacing, figure 4 to 6 cubic feet (4 to 6 Bags of AXIS Fine) per 1,000 square feet. 

3 x 4 spacing, figure 2 to 3 cubic feet (2 to 3 Bags of AXIS Fine) per 1,000 square feet.

Hanging Baskets & Rooftop Gardens: We recommend 20% by volume for best results in these specialty applications.  For offsite blending, mix 1 part AXIS (140 lbs. = 0.9 cu. yd.) with 9 parts soil mix to achieve 20% by volume.

Offsite Mixing or Pre-blending:  Use 70 lbs. per cubic yard to achieve 10% by volume in a mix for planting media, sand based systems or other blended mixes.  AXIS Regular should be used for the majority of all applications.  Use AXIS Fine for golf green rootzone construction.

Quantities for Individual Plantings

Application Rates and Worksheet to Apply 10% by Volume in the backfill.

# of Plants        Container        Pounds of       Volume of         Total       Added Available
                         or Plant Size    AXIS / plant     AXIS / plant

           Water per Plant*
__________
     
4” pot 
          .37 lbs. 
        2 cups
       _________     .39 lb. / 6 oz.

__________
       1 gal. 
          1.5 lbs. 
    1.5 quarts
 _________     1.5 lb. / 24 oz.
__________
      
 2 gal.    
       3 lbs. 
       3 quarts       _________     3.0 lb. / 1.5 quarts
__________

 5 gal.             7.5 lbs. 
       1.25 gal.      _________     7.5 lb. / .9 gal.

__________

  24” 
            3 lbs. 
        3 quart        _________     3.0 lb. / 1.5 quarts

__________

  48” 
          15 lbs. 
        2.5 gal.       _________     15  lb. / 1.8 gal.

__________

   6'
                 15 lbs. 
        2.5 gal.       _________     15  lb. / 1.8 gal.

__________

   8'
                 25 lbs. 
        1.0 bag 
 _________     25  lb. / 3.0 gal.

__________

 12'
                 60 lbs. 
      2.4 bags
 _________     60  lb. / 7.2 gal.

__________

  2" cal.            50 lbs.          2.0 bags
        _________     50  lb. / 6.0 gal.

__________

  3" cal.            75 lbs. 
      3.0 bags
 _________     75  lb. / 9.0 gal.

*Additional Available Water figures rely upon the amount of AXIS used per plant, AXIS Regular’s absorption rate of 114% 

it’s weight in water, and results from Labosport that state that 93% of the water AXIS absorbs is available to the plant.
For further assistance please call EnviroTech Soil Solutions, Inc., tollfree: 866-546-3722, 
or visit www.axisplayball.com
[image: image17.jpg]


 









Application Rates



          

   Page 14
Play Ball! Application Rates are 10% by volume. To apply; determine the square footage and depth to which you want to condition, apply at the rates listed per depth, and nail drag or incorporate to the proper depth. Apply additional product at the Standard Rate if areas between foullines and fences are skinned. For Infield Mixes blended and placed, determine the total cubic yards and blend per instructions. Requirements may vary according to field conditions.  

STANDARD RATE – 10% by volume

1” Deep - Apply 225 lbs. of Play Ball! per 1,000 square feet

2” Deep - Apply 450 lbs. of Play Ball! per 1,000 square feet


3” Deep - Apply 675 lbs. of Play Ball! per 1,000 square feet


4” Deep - Apply 900 lbs. of Play Ball! per 1,000 square feet


6” Deep - Apply 1300 lbs. of Play Ball! per 1,000 square feet

BASEBALL FIELDS

Infield Sq. Ft.      Tons 2” deep        Tons 4” deep
 
Tons 6” deep
90’ Bases


Grass Infield


11,550



2.6

      

5.2



7.8


Skinned Infield


18,300



4.1

      

8.2
         

12.3

80’ Bases


Grass Infield


  8,400



1.9

      

3.8



5.7


Skinned Infield


13,650



3.1

      

6.1



9.2

70’ Bases


Grass Infield


  6,800



1.5

      

3.0



4.5


Skinned Infield


10,700



2.4

      

4.8



7.2

60’ Bases


Grass Infield


  3,850

  

  .9

      

1.8



2.7


Skinned Infield

  
  6,700



1.5

      

3.0



4.5

SOFTBALL FIELDS

Infield Sq. Ft.
Tons 2” deep        Tons 4” deep
 Tons 6” deep

60’ Bases



  


Grass Foul Areas

  8,350



1.9

      

3.7



5.6

65’ Bases



  


Grass Foul Areas

  9,300



2.1

      

4.2



6.3
BLENDED and PLACED INFIELD MIX – 10% by volume


To produce one cubic yard of blended infield mix with 10% Play Ball!; mix 1 part Play Ball! 
Infield Conditioner (0.1 cubic yard = 70 pounds) with 9 parts (0.9 cubic yard) 
of infield soil. 


Play Ball! Regular should be used for the majority of applications.
www.axisplayball.com     [image: image18.jpg]KEnviroTech

Soil Solutions, Inc.






866-546-3722
